# SMI AI Node Handoff

# What This Is

The SMI AI node is a small Ubuntu computer connected to the SMIPACK machine area. It is being used to view machine status, show the HMI remotely, preview/record an IP camera, and prepare for AI fault detection.

This is a monitoring system. It should not control emergency stop or any safety-critical machine function.

# Current Hardware

Node PC: Lenovo ThinkCentre M715q class thin client

Boot drive: 256GB NVMe

Data/video drive: 512GB SSD mounted at /srv/smi-ai

Camera: Reolink RLC-522

Planned AI hardware: Coral Dual Edge TPU

# Current Addresses

Tailscale node IP: 100.87.194.41

Shop Wi-Fi/LAN node IP: 192.168.1.84

Machine/camera-side node IP: 192.168.0.20

Camera 1: 192.168.0.25

Machine/HMI: 192.168.0.1

# What Is Running

| Service | Port | Purpose |
| --- | --- | --- |
| SMI Web UI | 8000 | Main dashboard for phone/tablet/browser |
| SSH | 22 | Remote service/admin access |
| noVNC HMI viewer | 6080 | Browser view of machine HMI |
| go2rtc web/API | 1984 | Camera stream bridge |
| RTSP restream | 8554 | Camera restream |
| WebRTC | 8555 | Low-latency camera preview |

Main dashboard URLs:

LAN: http://192.168.1.84:8000

Tailscale: http://100.87.194.41:8000

# What The Dashboard Does Now

shows latest machine sample/status

can run in simulated mode when the machine is not connected

shows the HMI display when noVNC is running

shows the camera preview

has HMI and camera snapshot buttons

has operator buttons for FAULT and BAD PACK(S)

has a shutdown button with a confirmation popup

# Camera And Recording

Camera admin page: http://192.168.0.25

RTSP camera access stays on the machine/camera network

Video records to /srv/smi-ai/video

Camera snapshots save to /srv/smi-ai/camera_snapshots

Camera credentials should only live in /srv/smi-ai/config/cameras.env

The recording is a rotating buffer. About 400GB of video space gives roughly 6 days for one camera at about 6 Mbps.

# Normal Startup Check

From the node:

cd ~/AI-Fault-Detection
bash scripts/bringup_node_stack.sh

Without the machine connected:

bash scripts/bringup_node_stack.sh --simulate

This checks the web UI, machine data, camera access, and recording.

# IT Network Request

We want remote access through Tailscale and local dashboard access on the approved LAN.

Tailscale outbound access needed from the node:

TCP 443 outbound

UDP 41641 outbound

UDP 3478 outbound

optional TCP 80 outbound

Optional:

inbound UDP 41641 to the node can improve Tailscale direct connections, but it is not required.

If IT needs domain allowlisting:

login.tailscale.com

controlplane.tailscale.com

log.tailscale.com

console.tailscale.com

Tailscale DERP relay hostnames

References:

Tailscale firewall ports FAQ: https://tailscale.com/docs/reference/faq/firewall-ports

Tailscale firewall compatibility and workarounds: https://tailscale.com/docs/integrations/firewalls#firewall-compatibility-and-workarounds

# Firewall Intent

UFW should stay enabled.

Allowed over Tailscale:

SSH: 22

SMI Web UI: 8000

noVNC: 6080

go2rtc/camera services: 1984, 8554, 8555

Allowed on normal LAN:

SMI Web UI only: 8000

Do not broadly expose SSH, noVNC, or camera streaming ports on the live company LAN unless IT approves it.